MIMU310 Introduction to IE

Homework #2

Due Monday, September 19

Assembly Line Balancing

A new model toy is being assembled.  The table below defines the precedence relationships and element times for this assembly line.

| Element | Te | Immediate Predecessors |
| --- | --- | --- |
| 1 | 0.5 | ---- |
| 2 | 0.3 | 1 |
| 3 | 0.8 | 1 |
| 4 | 0.2 | 2 |
| 5 | 0.1 | 2 |
| 6 | 0.6 | 3 |
| 7 | 0.4 | 4,5 |
| 8 | 0.5 | 3,5 |
| 9 | 0.3 | 7,8 |
| 10 | 0.6 | 6,9 |

- Construct the precedence diagram for this job.

- If the ideal cycle time is to be 1.0 min, what is the theoretical minimum number of stations to minimize the balance delay?

- Determine the assignment of work elements to stations using the largest-candidate rule.  How many stations are required?

- Compute the balance delay.